## 1. Common Desktop PC Hardware Issues and Their Symptoms

| Hardware Issue | Typical Symptom |
| --- | --- |
| Faulty RAM | Computer does not boot, continuous beep codes, or random crashes. |
| Power Supply (SMPS) failure | PC does not power on or shuts down unexpectedly. |
| Hard Disk/SSD failure | Slow performance, boot errors, or "Operating System Not Found" message. |
| Graphics Card (GPU) issue | No display, screen artifacts, or distorted graphics. |
| CPU Overheating | Random shutdowns, frequent restarts, or system freezing during use. |

# 2. Troubleshooting Plan: Laptop Not Charging

### Step 1: Check the Power Outlet

- Plug another device into the same wall socket.

- Check: Confirm the outlet is supplying power.

### Step 2: Inspect the Charger

- Look for cuts, bent pins, or a damaged cable.

- Check: Ensure the charger and connector are not physically damaged.

### Step 3: Verify the Charging LED

- Connect the charger to the laptop.

- Check: See if the charging or power indicator LED turns on.

### Step 4: Check the Charging Port

- Inspect the laptop's charging port.

- Check: Look for dirt, loose connections, or broken pins.

### Step 5: Test with Another Compatible Charger

- Use a charger with the same voltage and current rating.

- Check: Determine whether the original charger is faulty.

### Step 6: Check the Battery

- If the battery is removable, remove it and inspect it for swelling or damage.

- Try powering the laptop using only the charger.

- Check: Determine whether the battery has failed.

### Step 7: Inspect the Power Connector

- Gently move the charging plug while connected.

- Check: If charging starts and stops, the DC jack may be loose or damaged.

### Step 8: Seek Hardware Repair

- If none of the above resolves the issue, the charging circuit or motherboard may require professional repair.

# 3. USB Charger Voltage Test (Multimeter)

Procedure

- Set the multimeter to DC Voltage (20V range or Auto Range).

- Plug the USB charger into a power outlet.

- Place the red probe on the USB +5V pin (VBUS).

- Place the black probe on the Ground (GND) pin.

- Read the voltage displayed on the multimeter.

### Example Observation

| Measured Voltage | Expected Range | Result |
| --- | --- | --- |
| 5.08 V DC (example reading) | 4.75 V – 5.25 V | Within the normal USB voltage range. |

Note: Replace the example value (5.08 V) with your actual measured value if you perform the test.

# 4. Testing the ATX 12V Rail with a Multimeter

Important: This test should only be performed by someone familiar with electrical safety. Avoid touching exposed metal contacts while the power supply is energised.

### Steps

- Disconnect the ATX power connector from the motherboard.

- Turn on the power supply using the appropriate method (for example, a PSU tester or by safely bridging the PS_ON pin to ground if you know the correct procedure).

- Set the multimeter to DC Voltage.

- Place the black probe on any black (Ground) wire.

- Place the red probe on a yellow (+12V) wire.

- Read and record the voltage.

### Example Observation

| Measured Voltage | Expected Range | Status |
| --- | --- | --- |
| 12.10 V DC (example reading) | 11.4 V – 12.6 V (ATX tolerance ±5%) | Normal |

Note: Replace the example value (12.10 V) with your own measurement if you perform the test.

### If the Voltage Is Below 11.5 V

A reading below 11.5 V may indicate:

- A failing or defective SMPS (Power Supply Unit).

- An overloaded power supply.

- Age-related degradation of the PSU.

- Internal component failure (such as failing capacitors).

A weak 12 V rail can cause symptoms such as:

- Random shutdowns.

- Unexpected restarts.

- Failure to boot.

- System instability, especially when the CPU or graphics card is under load.