# 1. Real-World Devices Using SMPS and UPS

## Devices Using SMPS (Switched Mode Power Supply)

| Device | Why it Uses SMPS |
| --- | --- |
| Desktop Computer | Converts AC power into stable DC voltages for the motherboard, CPU, and other components. |
| Laptop Charger | Converts mains AC into low-voltage DC to charge the laptop battery safely. |
| LED TV | Provides efficient and regulated DC power to the television's internal circuits. |

## Devices Using UPS (Uninterruptible Power Supply)

| Device | Why it Uses UPS |
| --- | --- |
| Desktop Computer | Prevents data loss and allows safe shutdown during power outages. |
| Wi-Fi Router | Keeps the internet connection running during short power cuts. |

# 2. Diagram of a Typical Desktop PC SMPS

AC Mains Input (230V AC)

│

▼

+------------------+

|   AC Input &     |

|      Filter      |

+------------------+

│

▼

+------------------+

|    Rectifier     |

| (AC → DC Output) |

+------------------+

│

▼

+------------------+

| High-Frequency   |

|   Switching      |

|   Circuit (PWM)  |

+------------------+

│

▼

+------------------+

| Transformer      |

| (Voltage Change) |

+------------------+

│

▼

+------------------+

| Rectifier &      |

| Output Filter    |

+------------------+

│

▼

+------------------+

| DC Outputs       |

| +3.3V            |

| +5V              |

| +12V             |

+------------------+

# 3. Power Connectors Used in a Desktop PC

5

| Connector | Devices it Powers | Safety Tip |
| --- | --- | --- |
| ATX 24-pin Motherboard Connector | Motherboard | Turn off and unplug the PC before connecting or removing the connector. |
| SATA Power Connector | SSDs, HDDs, DVD/Blu-ray drives | Insert gently in the correct orientation; never force it. |
| Molex Connector (4-pin) | Older hard drives, optical drives, fans | Hold the connector by its plastic housing when unplugging. |
| PCIe 6/8-pin Connector | Graphics Card (GPU) | Ensure the connector clicks into place before powering on the PC. |

# 4. Steps to Connect an SMPS in a New Gaming PC

- Install the SMPS

  - Place the SMPS inside the computer case.

  - Secure it using the four mounting screws.

- Connect Power to the Motherboard

  - Use the 24-pin ATX power connector.

  - Plug it into the motherboard until the locking clip clicks.

- Connect CPU Power

  - Use the 4+4-pin (8-pin) EPS CPU power connector.

  - Connect it to the CPU power socket near the processor.

- Connect the Graphics Card

  - Install the graphics card into the PCIe x16 slot.

  - Connect the required PCIe 6-pin or 8-pin power cable from the SMPS to the graphics card.

- Connect Storage Devices

  - Use SATA power connectors to connect SSDs, HDDs, or optical drives.

  - Also connect a SATA data cable from each storage device to the motherboard.

- Connect Additional Devices

  - Connect case fans, RGB controllers, or other accessories using SATA or Molex connectors if required.

- Check All Connections

  - Ensure every connector is fully inserted and locked.

  - Organize cables using cable ties to improve airflow.

- Power On and Test

  - Connect the AC power cable to the SMPS.

  - Turn on the SMPS switch.

  - Start the PC and verify that the motherboard, graphics card, and storage devices are detected correctly.