## 1. Common BIOS/POST Error Messages

| BIOS/POST Error Message | Hardware or Configuration Issue |
| --- | --- |
| CMOS Checksum Error – Defaults Loaded | CMOS battery may be weak, or BIOS settings have been reset/corrupted. |
| Keyboard Error or No Keyboard Present | Keyboard is not connected, faulty, or the USB/PS/2 port is not working. |
| No Bootable Device Found | Hard drive/SSD is not detected or there is no bootable operating system installed. |
| CPU Fan Error | CPU cooling fan is disconnected, faulty, or spinning too slowly. |
| Memory Test Failed | RAM is faulty, not properly seated, or incompatible with the motherboard. |

# 2. POST Beep Code Scenario (AMI BIOS)

### Problem

- Three repeating beeps

- No display on the monitor

### Typical Meaning (AMI BIOS)

For many AMI BIOS implementations, 3 short beeps usually indicate a memory (RAM) failure, such as faulty RAM or improperly installed RAM.

Note: Beep codes can vary slightly between BIOS versions and motherboard manufacturers, so the motherboard manual should always be checked for the exact meaning.

### Troubleshooting Steps

- Reseat the RAM

  - Turn off the PC and unplug it.

  - Remove the RAM modules.

  - Clean the contacts if necessary.

  - Reinsert them firmly into the memory slots.

- Test the RAM

  - Try one RAM module at a time.

  - Test each module in different memory slots.

  - Replace any module that is found to be faulty.

# 3. BIOS Setup Utility

This task requires access to a physical computer or virtual machine, so I cannot capture a BIOS screenshot myself.

### Steps

- Restart the computer.

- Press the BIOS key during startup (Delete, F2, F10, or Esc, depending on the system).

- Open the Main or System Information page.

- Record the following information:

  - Motherboard Manufacturer

  - BIOS Version

- Take a screenshot or photograph of the BIOS screen.

### Example

| Item | Example |
| --- | --- |
| Motherboard Manufacturer | ASUS |
| BIOS Version | 3602 |

(Replace the example values with the actual information from your system.)

# 4. CMOS Checksum Error – Defaults Loaded

### What It Means

The message "CMOS Checksum Error – Defaults Loaded" indicates that the BIOS detected that the data stored in the CMOS memory does not match the expected checksum. As a result, the BIOS loads its default settings to allow the computer to boot.

This commonly happens because:

- The CMOS battery is weak or depleted.

- BIOS settings were recently reset.

- A BIOS update or power interruption corrupted the stored settings.

### How to Resolve the Issue

- Restart the computer and enter the BIOS setup.

- Check that the date and time are correct.

- Reconfigure any custom BIOS settings (such as boot order or virtualization settings).

- Save the settings and restart the computer.

- If the error returns, replace the CMOS battery (typically a CR2032 coin-cell battery).

- After replacing the battery, enter the BIOS again, set the correct date, time, and required settings, then save and restart.

- If the problem still persists after replacing the battery, consider updating the BIOS (following the motherboard manufacturer's instructions) or checking the motherboard for hardware faults.

Expected Result: After replacing a weak CMOS battery and saving the BIOS settings, the error message should no longer appear during startup.